EXPERIMENT 2
GENERATION OF SQUARE WAVE USING PROTEUS

AIM: To write an assembly language program to generate square wave using AT89C51.

SOFTWARE REQUIRED: Proteus 8 software

ALGORITHM

Initialize program execution.

Set P2.0 HIGH and wait for the delay.

Clear P2.0 and wait for the same delay.

Repeat the sequence continuously to obtain a square wave.

PROGRAM

ORG 0000H
UP: SETB P2.0
    ACALL DELAY
    CLR P2.0
    ACALL DELAY
    SJMP UP

DELAY: MOV R4,#35
H1:    MOV R3,#255
H2:    DJNZ R3,H2
       DJNZ R4,H1
       RET
END

OUTPUT IMAGE

RESULT

Thus the program has been successfully verified and executed.